Department of Marine Sciences

School of Environment

University of the Aegean

Beach retreat estimator GUI

(Static models)

Manual

Beach Retreat Estimator 2

Introduction

The users of this GUI can estimate beach retreat according to static models (Edelman’s, Bruun’s and Dean’s) for particular environmental conditions (beach slope/bathymetry, wave height, and wave period) and sea level rise scenarios of their interest. The users can select all or some of these models to create an ensemble and then they can obtain the mean values (by selected models) of beach retreat and the polynomial equation that fits in the mean values of the retreat. This GUI is consisted of the exe files BRE_2a (for linear profiles) and BRE_2b (for natural profiles).

How to use the program

Once MCR is installed the user can run the platform by simply double-clicking on the BRE_2a.exe (for linear profiles) or BRE_2b (for natural profiles) (it might take some time to open).

In the panel named ‘Insert data’ (Figure 1), the user enters information on beach slope, wave height and wave period. He/she can also insert sea level rise scenarios pushing the button ‘Insert sea level rise scenarios’. Then an input dialog box appears with default values been already inserted which the user can use, or he/she can erase them and enter new values. The input dialog box with default values is shown in Figure 3. In the BRE_2b platform for natural profiles, there is a ‘Browse…’ button instead of the ‘Beach slope’ edit box (Figure 2). The push of the ‘Browse…’ button opens a new window through which the user can search in his/her computer for the txt profile file. This file should have two columns, the first column should be the cross-shore distance in meters and the second should be the elevation also in meters. This kind of file might be obtained by the EOF GUI.

Figure 1. The figure of the BRE_2a GUI (for linear profiles).

Figure 2. The figure of the BRE_2b GUI (for natural profiles).

Figure 3. Input dialog box with default sea level rise scenarios.

Once all parameters are determined, the user can run the models by using the buttons ‘Run’ inside the panel named ‘Run models’ (Figure 1, 2). The user has the option to view beach retreat estimations made by each model individually by pushing the button ‘View results’ below the pushbutton ‘Run’. When the ‘View results’ button is pressed, a message box opens with a table consisted by 2 columns. The first column is the sea level rise scenarios (in m) and the second is the beach retreat estimations (in m) made by the model. An example of this message box is shown in Figure 4. The user can select models to create an ensemble, by clicking in the corresponding checkboxes (Figure 5).

Figure 4. An example of message box with results of the Edelman model.

Once the models comprising the ensemble are selected, the user use the button ‘View equation’; then, a polynomial equation describing beach retreat estimations appears in the text fields below the button. An example is shown in Figure 5. This equation have the form: s = p1a2 +p2a +p3, where s: is the beach retreat, p1, p2 and p3: are the polynomial coefficients and a: is the sea level rise. The value of R2 is also given in the text field.

Figure 5. An example of the resulted equation.

With the press of the pushbutton ‘Plot results’, the polynomial equations of all selected models are calculated and plotted together with the polynomial equation that fits in the mean values of beach retreat. An example of a plot result is depicted in Figure 6.

Figure 6. An example of a plotting result.

From the equation resulted by the program, the range of beach retreat due to a specific value of sea level rise can be easily calculated. This program gives the user the possibility to calculate the range of retreat only with a click on the pushbutton calculate inside the panel ‘Calculate the retreat for a specific sea level rise scenario’. An example is given in Figure 7.

Figure 7. An example of beach retreat calculation for a specific sea level rise scenario.

The output files of this platform are:

(Output files are saved in the same folder with the platform’s exe file, replacing pre-existing files)

extpr.txt (only for natural profiles): is the extended profile of the initial profile given by the user. First column is cross-shore distance (in m) and second column is elevation (in m).

sEdel.txt: is a one column matrix with beach retreat projections made by Edelman model.

sBruun.txt: is a one column matrix with beach retreat projections made by Bruun model.

sDean.txt: is a one column matrix with beach retreat projections made by Dean model.

slrs.txt: is a one column matrix that contains the sea level scenarios entered by the user.

total_table.txt: is a table with 5 rows and columns equal with the number of the sea level rise scenarios entered by the user. Contains the beach retreat projections made by Edelman (1st row), Bruun (2nd row) and Dean (3rd row). Each column corresponds to a different sea level rise scenario.

path.txt: contains the path and the filename of the imported profile.

Beach retreat estimations.tiff:  is the plotting figure of the polynomial equations of all selected models together with the polynomial equation that fits in the mean values of beach retreat projections of the models selected by the user.

References

Bruun, P., 1962, Sea level rise as a cause of shore erosion, Journal of Waterway, Harbors Division. ASCE 88:117-130.

Bruun, P., 1983, Review of conditions for uses of the Bruun rule of erosion, Coastal Engineering 7(1):77-89.

Bruun, P., 1988, The Bruun Rule of erosion by sea level rise: A discussion on large-scale two- and three-dimensional usages, Journal of Coastal Research 4(4):622-648.

Dean, R. G., 1991, Equilibrium beach profiles: characteristics and applications, J. Coastal Research 7(1):53– 84.

Edelman, T., 1972, Dune erosion during storm conditions, in: Proceedings of the 13th International Conference on Coastal Engineering ASCE, pp. 1305-1312.