Department of Marine Sciences

School of Environment

University of the Aegean

Beach retreat estimator GUI

(Dynamic models)

Manual

Beach Retreat Estimator 3

Introduction

The users of this GUI can estimate beach retreat according to dynamic models (Leont’yev and SBEACH) for particular environmental conditions (beach slope, wave height, wave period, sediment size) and sea level rise scenarios of their interest. The models should run first without a sea level rise scenario to be considered. In this way the profile evolution caused only by environmental forcing is estimated. Then the new profile is used as an initial profile for the assessment of sea level rise effects. This GUI is consisted of the exe files BRE_3a (for linear profiles) and BRE_3b (for natural profiles).

How to use the program

Once MCR is installed the user can run the platform by simply double-clicking on the BRE_3a.exe (for linear profiles) or BRE_3b (for natural profiles) (it might take some time to open). The following exe files compiled in FORTRAN should be in the same folder with platform’s exe file.

Exe files compiled in FORTRAN:

leo_first run: Leont’yev run without sea level rise

leo_slr run: Leont’yev run with sea level rise

sbeach_first run: SBEACH run without sea level rise

sbeach_slr run: SBEACH run with sea level rise

Figure 1. The figure of the BRE_3a GUI (for linear profiles).

Figure 2. The figure of the BRE_3b GUI (for natural profiles).

In the panel named ‘Insert environmental conditions’, the user enters information on beach slope, wave height, wave period, wave angle and sediment size (Figure 1). In the BRE_3b platform for natural profiles, there is a ‘Browse…’ button instead of the ‘Beach slope’ edit box (Figure 2). The push of the ‘Browse…’ button opens a new window through which the user can search in his/her computer for the txt profile file. This file should have two columns, the first column should be the cross-shore distance in meters and the second should be the elevation also in meters. This kind of file might be obtained by the EOF platform (see section 6).

Once all parameters are determined, the user can enter the simulation time of his/her desire by pushing the button ‘Insert simulation time’, then he/she can run the models by using the buttons ‘Run Leont’yev’ or ‘Run SBEACH’ inside the panel named ‘Beach profile evolution without sea level rise’ (Figure 1,2). A waitbar appears with the title ‘Please wait...’ (Figure 3), the user should wait (or he/she can cancel the process by pushing the button "cancel" inside the waitbar window) for the process to be completed, then a message informs the user that the model have ran successfully. During the process, instability of the model may cause the process to stop, then a warning message appears, informing the user that the model stopped running and how many calculations have been completed successfully. The user can calculate coastline migration (Retreat (-) or Accretion (+)) by pushing the button ‘Calculate coastline migration’, then the result appears in the edit box ‘Retreat (-) or Accretion (+)’. The user has also the options to view the video of the profile evolution and to plot initial and final profile (Figure 4) by pushing the buttons ‘View the video’ and ‘Plot initial and final profile’ respectively.

Figure 3. The waitbar that appears during the model’s run.

Once the resulted profile by the environmental forcing is been estimated, sea level rise effects can be also assessed. The user enters the sea level rise scenario of his/her interest (in the edit box ‘Sea level rise’), the desired simulation time (by pushing the button ‘Insert simulation time’) and then he/she can run the models by using the buttons ‘Run Leont’yev’ or ‘Run SBEACH’ inside the panel named ‘Beach profile evolution with sea level rise’ (Figure 1, 2). Once the model process has been completed successfully the user has the options to calculate the retreat of the coastline, to view the video of profile evolution and to plot initial and final profiles by pushing the buttons ‘Calculate coastline migration’, ‘View the video’ and ‘Plot initial and final profile’ respectively.

Figure 4. An example of initial and final profile plotting.

Figure 5. An example frame of profile evolution video.

The output files of this platform are:

extpr.txt (only for natural profiles): is the extended profile of the initial profile given by the user. First column is cross-shore distance (in m) and second column is elevation (in m).

Output files from Leont’yev runs without sea level rise:

inputs_leo_fr.txt: file with user’s input data

e.g.  [10   2   6    0.8    220 45]

Where 10 the beach slope (for natural profiles, beach slope is estimated by the mean slope of the swash zone), 2 wave height (in m), 6 wave period (sec), 0.8 grain size (in mm) 220 the number of beach profile points defined by the bed slope (if the profile starts at 20 m water depth and reaches to 10 m elevation above the sea level) and 45 is the wave angle.

in_bathy_leo.txt: the initial profile.

time_leo_fr.txt: simulation time set by the user.

bed_leo_fr.txt: the final model output with the first column showing the x axis, the second column the final profile and the third the profile of the previous run.

all_bathy_leo_fr.txt: each column contains the topographic elevation after each iteration For example, for time 40 (40*200=8000sec) the file contains 40 columns.

situation_leo_fr.txt: the file contains information on the condition of the run. For example, the first value shows if the model ran correctly (value 0) or stopped (value 1). The second value shows if the model gives NaN values for the profile, the 3rd value shows if the model gives zero values for the profile, the 4th value shows if  the model gives ****** values for the profile and the 5th value shows the iteration that showed instability and stopped. If the model run successfully to the end for e.g. the default (40) time then the values will be:

0

0

0

0

40

If the 20η time the model stopped and gave NaN values, then ‘Warning’ appears (‘The model stopped running, it have run 19 times’) and the values on the table will be:

1

1

0

0

20

Leont_fr.tiff: figure file with the initial and final profile

leont_fr.avi: video file.

Output files from Leont’yev runs with sea level rise:

inputs_leo_slr.txt: file with user’s input data

e.g.  [10   2   6    0.8    220 45]

Where 10 the beach slope (for natural profiles, beach slope is estimated by the mean slope of the swash zone), 2 wave height (in m), 6 wave period (sec), 0.8 grain size (in mm) 220 the number of beach profile points defined by the bed slope (if the profile starts at 20 m water depth and reaches to 10 m elevation above the sea level) and 45 is the wave angle.

slr_scenario_leo.txt: file containing one value (user input for sea level rise).

time_leo_slr.txt: simulation time set by the user.

bed_leo_slr.txt: the final model output with the first column showing the x axis, the second column the final profile and the third the profile of the previous run.

all_bathy_leo_slr.txt: each column contains the topographic elevation after each iteration For example, for time 40(40*200=8000sec) the file contains 40 columns.

situation_leo_slr.txt: the file contains information on the condition of the run. For example, the first value shows if the model ran correctly (value 0) or stopped (value 1). The second value shows if the model gives NaN values for the profile, the 3rd value shows if the model gives zero values for the profile, the 4th value shows if  the model gives ****** values for the profile and the 5th value shows the iteration that showed instability and stopped. If the model run successfully to the end for e.g. the default (40) time then the values will be:

0

0

0

0

40

If the 20η time the model stopped and gave NaN values, then ‘Warning’ appears (‘The model stopped running, it have run 19 times’) and the values on the table will be:

1

1

0

0

20

Leont_slr.tiff: figure file with the initial and final profile

leont_slr.avi: video file.

Output files from SBEACH runs without sea level rise:

inputs_SB_fr.txt: file with user’s input data

e.g.  [10   2   6    0.8    220 45]

Where 10 the beach slope (for natural profiles, beach slope is estimated by the mean slope of the swash zone), 2 wave height (in m), 6 wave period (sec), 0.8 grain size (in mm) 220 the number of beach profile points defined by the bed slope (if the profile starts at 20 m water depth and reaches to 10 m elevation above the sea level) and 45 is the wave angle.

in_bathy_SB.txt: the initial profile.

time_SB_fr.txt: simulation time set by the user.

bed_SB_fr.txt: the final model output with the first column showing the x axis, the second column the final profile and the third the profile of the previous run.

all_bathy_SB_fr.txt: each column contains the topographic elevation after each iteration For example, for time 40(40*200=8000sec) the file contains 40 columns.

situation_SB_fr.txt: the file contains information on the condition of the run. For example, the first value shows if the model ran correctly (value 0) or stopped (value 1). The second value shows if the model gives NaN values for the profile, the 3rd value shows if the model gives zero values for the profile, the 4th value shows if  the model gives ****** values for the profile and the 5th value shows the iteration that showed instability and stopped. If the model run successfully to the end for e.g. the default (40) time then the values will be:

0

0

0

0

40

If the 20η time the model stopped and gave NaN values, then ‘Warning’ appears (‘The model stopped running, it have run 19 times’) and the values on the table will be:

1

1

0

0

20

SBEACH_fr.tiff: figure file with the initial and final profile

sbeach_fr.avi: video file.

Output files from SBEACH runs with sea level rise:

inputs_SB_slr.txt: file with user’s input data

e.g.  [10   2   6    0.8    220 45]

Where 10 the beach slope (for natural profiles, beach slope is estimated by the mean slope of the swash zone), 2 wave height (in m), 6 wave period (sec), 0.8 grain size (in mm) 220 the number of beach profile points defined by the bed slope (if the profile starts at 20 m water depth and reaches to 10 m elevation above the sea level) and 45 is the wave angle.

slr_scenario_SB.txt: file containing one value (user input for sea level rise).

time_SB_slr.txt: simulation time set by the user.

bed_SB_slr.txt: the final model output with the first column showing the x axis, the second column the final profile and the third the profile of the previous run.

all_bathy_SB_slr.txt: each column contains the topographic elevation after each iteration. For example, for time 40(40*200=8000sec) the file contains 40 columns

situation_SB_slr.txt: the file contains information on the condition of the run. For example, the first value shows if the model ran correctly (value 0) or stopped (value 1). The second value shows if the model gives NaN values for the profile, the 3rd value shows if the model gives zero values for the profile , the 4th value shows if  the model gives ****** values for the profile and the 5th value shows the iteration that showed instability and stopped. If the model run successfully to the end for e.g. the default (40) time then the values will be:

0

0

0

0

40

If the 20η time the model stopped and gave NaN values, then ‘Warning’ appears (‘The model stopped running, it have run 19 times’) and the values on the table will be:

1

1

0

0

20

SBEACH_slr.tiff: figure file with the initial and final profile

sbeach_slr.avi: video file

References

Larson, M., and Kraus, N. C., 1989, SBEACH: numerical model for simulating storm-induced beach change; report 1: empirical foundation and model development, Technical Report - US Army Coastal Engineering Research Center 89-9.

Leont'yev, I. O., 1996, Numerical modelling of beach erosion during storm event, Coastal Engineering 29(1-2):187-200.

Leont'yev, I. O., 1997, Short-term shoreline changes due to cross-shore structures: a one-line numerical model, Coastal Engineering 31(1-4):59-75.