Department of Marine Sciences

School of Environment

University of the Aegean

Wave Estimation GUI

Manual

Wave Estimation

Introduction

Wind and waves are closely related. When wind blows over water, friction pulls the surface water along in the same direction. The water gains energy from the wind and waves are formed because the water is being compressed by the wind. There are three types of wind wave growth:

Fully developed sea: When the wind has imparted its maximum energy to the waves, the sea is said to be fully developed. In brief, the waves cannot grow any higher than the maximum value for that wind speed.

Fetch limited sea: When the fetch length is too short, the wind is not in contact with the waves over a distance sufficient to impart the maximum energy to the waves. The ranges of frequencies and wave heights are therefore limited, and the wave heights are less than those of a fully developed sea. The process of wave generation is cut off before the maximum energy has been imparted to the waves and the fetch is in a steady state. This leads to the conclusion that for every wind speed, a minimum fetch distance is required for the waves to become fully developed, and that if this minimum fetch requirement is not met, the sea is fetch limited.

Duration limited sea: When  the wind  has  not  been  in  contact  with  the  waves  long enough,  it  has  had  insufficient  time  to  impart  the maximum energy to the waves, and the growth of the frequency range and wave heights ceases before the fully developed state of the sea has commenced. Such a situation is known as a duration time limited sea. This leads to the conclusion that for every wind speed, a minimum duration time is required for the waves to become fully developed; and that if this minimum duration time requirement is not met, the sea is duration time limited.

The JONSWAP-PM method

The JONSWAP-PM method estimates significant wave height and period (Hs, Tp) in open sea, from wind velocity, wind duration and effective fetch length, using the JONSWAP and the Pierson-Moskowitz spectrums. To apply this wave prediction procedure, first the adjusted wind speed UA must be determined given by

|  | [1] |
| --- | --- |

where UA and U10 are both in meters per second.

The JONSWAP spectrum is based on the fetch-limited conditions. Then the significant height (Hs) and peak period (Tp) can be calculated from:

|  | [2] |
| --- | --- |
|  | [3] |

if this inequation is valid:

The values determined from Eqs. (2) and (3) use only the wind speed and fetch and are thus only for the fetch-limited condition. Limiting wind duration would be calculated from:

|  | [4] |
| --- | --- |

were tD is the wind duration in seconds. If the actual duration is greater than the duration calculated from Eq. (4) the wind generation process is fetch limited and the results from Eqs. (2) and (3) are the predicted significant height and peak period. If the actual duration is less, the process is duration limited. Using the actual duration, a new effective fetch is calculated from Eq. (4) and, with this new fetch value, the significant height and peak period can be calculated from Eqs. (2) and (3). If the process is not limited (Fully developed sea) then:

|  | [5] |
| --- | --- |

In this case the Pierson-Moskowitz spectrum is used and significant height and peak period are calculated from:

|  | [6] |
| --- | --- |
|  | [7] |

The significant wave period Ts is calculated from:

|  | [8] |
| --- | --- |

Effective wave can be calculated from (Borah and Balloffet, 1985):

|  | [9] |
| --- | --- |
|  | [10] |

This GUI can estimate significant wave height and period (Hs, Tp) in open sea, from wind velocity and wind fetch, using JONSWAP method. This GUI is consisted of the exe file Wave_forecasting.

How to use the program

Once MCR is installed the platform can run by simply double-clicking on the Wave_forecasting.exe, then the following screen appears (it might take some time to open):

Figure 1. The figure of the Wave_forecasting GUI.

In the panel named ‘Insert data’ (Figure 1), the user enters information on wind fetch (the length of water over which a given wind has blown) and the z vertical distance of the wind gauge above sea level. The user can calculate significant and effective wave characteristics. In order to calculate significant wave the user should also enter information about the wind velocity measured at z distance from the wind blow and the wind duration. Once all parameters are determined, the user can calculate significant wave height and period by pushing the button ‘Calculate Hs, Ts’, then the results appear in the edit boxes ‘Significant wave height’ and ‘Significant wave period’ inside the panel named ‘Significant wave’ (Figure 1 and 2). The type of wind wave growth is also shown in the text field below the edit boxes inside the panel ‘Significant wave’ (Figure 2).

Figure 2. An example of significant wave results.

Figure 3. The window that opens in order to give a filename and save the results.

The user can save the results by pushing the button ‘Save results’ then a new window opens where he/she can give a filename (Figure 3). The input and output values are saved as an excel file (Table 1) in the output directory given by the user.

Table 1. An example of the output file with the resulted characteristics of significant wave.

|  |  |  |  | INPUTS |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Fetch (m) |  |  |  |  | 90000 |  |
| Vertical distance of the gauge (m) |  |  |  |  | 15 |  |
| Wind velocity (m/sec) |  |  |  |  | 11 |  |
| Wind duration (sec) |  |  |  |  | 36000 |  |
|  |  |  |  | RESULTS |  |  |
| Significant wave height (m) |  |  |  |  | 1,93 |  |
| Significant wave period (sec) |  |  |  |  | 5,86 |  |
| Situation of the sea |  |  |  |  | Fetch limited |  |

In order to calculate effective wave the user should also browse a txt file (by pushing the button "Browse input file" in the panel "Effective wave") that will include information about the wind frequency (2nd column), the mean wind velocity (3rd column) and the wind duration (4th column) per Beaufort (1st column). This kind of file might be obtained by the "Wind data analysis" GUI. Then effective wave can calculated by pushing the button "Calculate Effective Wave Characteristics". The results appear in the edit boxes ‘Effective Wave Height’ and ‘Effective Wave Period’ and the table next to them is filed by the frequency, the wave height and period per Beaufort category resulted by the application of JONSWAP method for every  Beaufort category  (Figure 4). The table and the effective wave characteristics can be saved in an excel file by pushing the button "Save results", inside the panel "Effective Wave" (Figure 1 and 4).

Figure 4. An example of effective wave results.

References

Borah D.K. and Balloffet A., 1985, Beach evolution caused by littoral drift barrier, J. of Waterway, Port, Coastal and Ocean Eng., ASCE, 111(4): 645-660.

Hasselmann, K., Ross, D.B., Muller, P., Sell, W., 1976, A Parametric Wave Prediction Model, Journal of Physical Oceanography 6, pp. 200-228.

Κουτίτας, Χ.Γ., 1994,  Εισαγωγή στην Παράκτια Τεχνική και τα Λιμενικά Έργα, Θεσσαλονίκη, Εκδόσεις Ζήτη.

Komar, P. D., 1998, Beach Processes and Sedimentation, Prentice Hall, N.J., USA, pp. 544.