Department of Marine Sciences

School of Environment

University of the Aegean

Wind Data Analysis GUI

Manual

Wind Data Analysis

Introduction

1. General characteristics

Wind is air in motion with respect to the surface of the earth. The general characteristics of the wind are the direction, from which it originates and the velocity/speed.

1.1 Wind direction

It is important to know how wind direction is recorded and reported. Wind direction is the compass direction from which the wind blows. For example, a northerly wind blows from the north to the south. The wind direction is expressed in degrees (0° to 360°) starting from the North and progressing clockwise (the point of 0° is the same with the point of 360°). 8 main and 8 secondary directions are recognized as depicted in Figure 1 and on Table 1.

Figure 1. 8 main and 8 secondary directions.

Table 1. The 8 main directions.

| Wind | Main direction | Direction-sector |
| --- | --- | --- |
| North | 0˚ | 338-23˚ |
| Northeast | 45˚ | 23-68˚ |
| East | 90˚ | 68-113˚ |
| Southeast | 135˚ | 113-158˚ |
| South | 180˚ | 158-203˚ |
| Southwest | 225˚ | 203-248˚ |
| West | 270˚ | 248-293˚ |
| Northwest | 315˚ | 293-338˚ |

1.2 Wind velocity (or intensity)

Wind velocity is the distance that an air mass covers in the unit of time. The wind velocity is affected by the different temperatures of atmospheric layers. The velocity at the height of 10m above sea level has been defined as a representative velocity. So if our measurements are recorded at a different height we need to convert the wind velocity from the height of measurements to the height of 10 m above sea level, using the following equation:

where U(z) is the wind velocity at the height of the measurements (z) and U10 is the wind velocity at the height of 10 m.

There are several units of measurements such as: meters per second (m/sec), kilometers per hour (km/h), miles per hour (miles/h) and knots (knots = 1,852 m/h). The correlation between them is the following:

1 m/s = 3.6 km/h = 1.944 knots = 2.237 miles/hour.

Historically, the Beaufort wind force scale provides an empirical description of wind speed based on observed sea conditions. The Beaufort scale was based on the equation:

U10 = 0.836 B3/2,

where B is Beaufort Number and U10 is the wind speed in meters per second at a height of 10 meters.

Table 2. Correlation between Beaufort scale and wind velocity according with the National Meteorological Service.

| Wind intensities in Beaufort |  | Wind velocity |  |  |  |
| --- | --- | --- | --- | --- | --- |
| B | Wind situation | m/s | km/h | knots | miles/h |
| 0 | Calm | 0-0.2 | < 1 | < 1 | <1 |
| 1 | Light air | 0.3-1.5 | 1-5 | 1-3 | 1-3 |
| 2 | Light breeze | 1.6-3.3 | 6-11 | 4-6 | 4-7 |
| 3 | Gentle breeze | 3.4-5.4 | 12-19 | 7-10 | 8-11 |
| 4 | Moderate breeze | 5.5-7.9 | 20-28 | 11-16 | 13-18 |
| 5 | Fresh breeze | 8.0-10.7 | 29-38 | 17-21 | 19-24 |
| 6 | Strong breeze | 10.8-13.8 | 39-49 | 22-27 | 25-31 |
| 7 | Moderate gale | 13.9-17.1 | 50-61 | 28-33 | 32-38 |
| 8 | Fresh gale | 17.2-20.7 | 62-74 | 34-40 | 39-46 |
| 9 | Strong gale | 20.8-24.4 | 75-88 | 41-47 | 47-54 |
| 10 | Whole gale | 24.5-28.4 | 89-102 | 48-55 | 55-63 |
| 11 | Storm | 28.5-32.6 | 103-117 | 56-63 | 64-74 |
| >=12 | Hurricane | >= 32.7 | >= 118 | >= 64 | >=75 |

1.3 Windrose

A wind rose is a graphic tool used by meteorologists to give a succinct view of how wind speed and direction are typically distributed at a particular location. Presented in a polar coordinate grid, the wind rose shows the frequency of winds blowing from particular directions. The length of each spoke around the circle is related to the frequency that the wind blows from a particular direction per unit time. Each concentric circle represents a different frequency, emanating from zero at the center to increasing frequencies at the outer circles. A wind rose plot may contain additional information, in that each spoke is broken down into color-coded bands that show wind speed ranges. Wind roses typically use 8 or 16 cardinal directions, such as north (N), NNE, NE, etc., although they may be subdivided into as many as 32 directions.

Figure 2. Windrose of wind data from the meteorological station in the University Hill.

Wind data consist records of the wind every half an hour or every hour and include the date and time of the records, the wind velocity and the wind direction.  The analysis of the wind data includes:

- Conversion of the velocity units in m/s (if the measurements are made in a different unit of measurement).

- Reduction of the velocity from the height of measurements to the height of 10 m above sea level (U10).

- Estimation for each of the 8 main direction-sector and for each Beaufort number, of the wind frequency, mean wind velocity and mean wind duration

This GUI is consisted of the exe file Wind_analysis.

How to use the program

Once MCR is installed the platform can run by simply double-clicking on the Wind_analysis.exe, then the following screen appears (it might take some time to open):

Figure 3. The figure of the Wind_analysis GUI.

Figure 4. An example of an input txt file.

First of all the user should browse a txt file that will include data of wind velocity and direction measurements. The txt file should have the format shown in Figure 4, where 1st column is the day of the measurement, 2nd is the month, 3rd is the year, 4th is the hour, 5th is the minutes, 6th is the seconds, 7th is the wind velocity and 8th is the wind direction. The user should also specify the units of measurements of the wind velocity (in the panel "Specify the units of wind velocity") and the height of the gauge above sea level (in the panel "Vertical distance of the gauge") (Figure 3).

In the panel "General analysis" the user can select one of the 8 main direction-sectors and calculate the wind characteristics (by pushing the button "Calculate wind characteristics") which are the wind frequency, mean wind velocity and mean wind duration for each Beaufort number. The results are shown in the table below the button "Calculate wind characteristics" (Figure 3). The user can save the resulted table by giving a filename in the edit box "Enter filename" and by pushing the button "Save the resulted table" (Figure 3). The user can also plot the windrose for the selected direction-sector, by pushing the button "Plot windrose" inside the panel "Create windrose" (Figure 3). He/she has the option to give a title to the windrose by entering the desired title in the edit box "Title" before pushing the button "Plot windrose".

The option of a specified analysis is also given. The user can give a specific range of wind directions (in the panel "Insert a range of wind directions") and a specific range of wind velocities (in the panel "Insert a range of wind velocities") and calculate the wind frequency, mean wind velocity and mean wind duration by pushing the button "Calculate wind characteristics"  in the panel "Specified analysis". The results are shown in the table below (Figure 3). The user can save the resulted table by giving a filename in the edit box "Enter filename" and by pushing the button "Save the resulted table" (Figure 3). The user can also plot the windrose for the specified range of wind directions, by pushing the button "Plot windrose" inside the panel "Create windrose" (inside the panel "Specified analysis") (Figure 3). He/she has the option to give a title to the windrose by entering the desired title in the edit box "Title" before pushing the button "Plot windrose".